---- INITIAL TICKET OPENING ----

NON-ISP RELATED

--Gateway Alert--

*Subject Template: Property Name | Offline Server | Ticket Number

*Body Template:

Issue Reported: The Gateway at (Site Name) – is offline.

Equipment Location:  The Gateway is located in the (insert Gateway location from SNAPx).

Outage Impact: Due to the Gateway being offline, guests will be unable to connect to the internet.

Troubleshooting Step Performed:

Opened a ticket for further investigation, Severity set to High.

Current Status Update: The Gateway is offline.

Next Steps:

- Will run remote tests for 2 minutes.

  - Result: 100% packet loss

- Will attempt to remotely power cycle the Gateway.

- Contact the property to confirm power to the equipment

- Check the physical status of the Gateway.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Assign T3 for remote troubleshooting

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

====================================

--Firewall Alert--

*Subject Template: Property Name | Offline Firewall | Ticket Number

*Body Template:

Issue Reported: The Firewall at (Site Name) – is offline.

Equipment Location:  The Firewall is located in the (insert Firewall location from SNAPx).

Outage Impact: Due to the Firewall being offline, guests will be unable to connect to the internet.

Troubleshooting Steps Performed:

Opened a ticket for further investigation, Severity set to High.

Current Status Update: The Firewall is offline.

Next Steps:

- Will run remote tests for 2 minutes.

- Will attempt to remotely power cycle the Firewall.

- Contact the property to confirm power to the equipment

- Check the physical status of the Firewall.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Assign T3 for remote troubleshooting

* Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

====================================

--Wireless Controller Alert--

*Subject Template: Property Name | Offline Wireless Controller | Ticket Number

*Body Template:

Ticket number:

Issue Reported: The Wireless Controller at (Site Name) – is offline.

Equipment Location: Wireless Controller is located in the (insert WLC location from SNAPx)

Outage Impact: Due to the Wireless Controller being offline, guests near the location, may experience limited or no connectivity.

Troubleshooting Steps Performed:

Opened a ticket for further investigation, Severity set to High.

Current Status Update:  Wireless Controller is offline.

Next Steps:

- Will make an attempt to power cycle the Access Point Controller via switch POE port.

- Will keep the access point Controller offline for 5 minutes and power cycle.

- Contact Site for assistance in physically rebooting the access point.

- Once the physical reboot is completed and the access point Controller is still down, NOC will set all access points to Stand-alone mode.

- Assign T3 for remote troubleshooting.

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

====================================

--Switch Alert--

*Subject Template: Property Name | Offline Switch/es | Ticket Number

*Body Template:

Ticket number:

Issue Reported: Switch/es in (Site Name)– is/are offline.

Equipment Location: The Switch/es is/are located in the (insert Switch/es location from SNAPx).

Outage Impact: Due to the Switch/es being offline, guests near in the said area may experience limited or no connectivity.

Troubleshooting Steps Performed:

- Opened a ticket for further investigation, Severity set to High.

Current Status Update: Switch/es is/are offline

Next Steps:

- Contact the site to confirm the power of the equipment.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Once the physical reboot is completed, and the Switch remains offline, we will attempt to move the switch uplink cable to another port.

- Assign T3 for remote troubleshooting.

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

====================================

--Virtual Controller AP Alert--

*Subject Template: Property Name | Offline AP/s | Ticket Number

*Body Template:

Ticket number:

Issue Reported: The AP/s in (Site Name) – is/are Offline

Equipment Location: AP/s is/are located in the (insert AP location from SNAPx)

Outage Impact: Due to the AP/s being offline, guests near in the said area may experience limited or no connectivity.

Troubleshooting Steps Performed:

- Opened a ticket for further investigation, Severity set to High.

Current Status Update: AP/s is/are still offline

Next Steps:

- Attempted to remotely power down access point via switch POE port.

  - Kept the AP R700 offline for 10 minutes and cycled power back however the access points remained offline.

- Contact Site for assistance in physically rebooting the access point.

- Once the physical reboot is completed and the access point/s is/are still down, we will need to test the offline access point directly to the Switch.

- Assign T3 for remote troubleshooting.

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

EMAIL OUTAGE NOTIFICATION TEMPLATE

(OFFLINE ISP)

A. OFFLINE ISP

Non-HILTON Subject: <SiteNameONLY> | Possible Offline ISP | <Case umber>

HILTON Subject: <Site Name & INN Code> | Possible Offline ISP | <Case Number>

--

Ticket number:

Date/Time of Event:

Issue Reported: The ISP (ISP name) circuit in (Site Name) - is being reported as Offline.

Equipment Location: The ISP ______ circuit is located in the (Floor #, IDF/MDF, Room Number)

Outage Impact: Due to the ISP ______ circuit being Offline, Guests/Staff are unable to connect to the internet.

Troubleshooting Steps Performed:

- Opened a ticket for further investigation, Severity set to Critical.

Current Status Update: The internet service provider’s modem/gateway is offline.

Next Steps:

- Will continuously run a remote test on both the internet service provider’s router and the Nomadix/Mikrotik/Meraki IP Address (for 3 minutes)

  - Result = 100% packet loss

- Contact ISP _____ for possible outages in the area.

- Contact property to confirm power to the equipment.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Assign T3 for remote troubleshooting.

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

B. OFFLINE ISP- ENTERPRISE

Ticket number:

Date/Time of Event:

Issue Reported: The ISP (ISP name) circuit in (Site Name) – has been reported offline.

Equipment Location: The ISP ______ circuit is located in the MDF.

Outage Impact: Due to the ISP ______ circuit being Offline, tenants are unable to connect to the internet.

Troubleshooting Steps Performed:

- Opened a ticket for further investigation, Severity set to Critical.

Current Status Update: The internet service provider’s modem/gateway is possibly Offline.

Next Steps:

- Will Continuously run a remote test on both the internet service provider’s router and the Nomadix/Mikrotik/Meraki IP Address (for 3 minutes)

  - Result = 100% packet loss

- Contact ISP _____ for possible outages in the area.

- Contact property to confirm power to the equipment.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Assign T3 for remote troubleshooting.

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).

ESM MAILBOX

SAMPLE SCENARIOS:

1st – outage notification – SENT 10am – ESM

=============

2nd – (within 2 hours – NONE) – SENT BY SYSTEM 12pm

3rd – (within 2 hours – YES) – agent sent email 1pm

4th - (within 2 hours – NONE) – SENT BY SYSTEM 3pm

5th - (within 2 hours – YES) – agent sent email 4:30pm

Ticket closure template

Template for no updates after 2/3 hours

-- Equipment Alert (edit equipment) —

*Subject Template: Property Name | Server | Ticket Number

*Body Template:

Issue Reported: The Gateway in (Site Name) – has been triggering alerts that could indicate that the _________ equipment is offline.

Equipment Location:  The Gateway is located in the (insert Gateway location from SNAPx).

Outage Impact: Due to the Gateway being possibly offline, guests will be unable to connect to the internet.

Troubleshooting Step Performed:

Opened a ticket for further investigation, Severity set to High.

Current Status Update: The Gateway is possibly offline.

Next Steps:

- Will run remote tests for 2 minutes.

  - Result: 100% packet loss

- Will attempt to remotely power cycle the Gateway.

- Contact the property to confirm power to the equipment

- Check the physical status of the Gateway.

- Attempt physical reboot/power cycle and trace cable uplinks.

- Assign T3 for remote troubleshooting

Another update will be sent in 2 hours (Omni) ; 3 hours (all other Brands).